LIVE SWITCH DATA

Available Ports

Feature Reference

How many switch ports are free right now — copper and fiber, live.

Part of the RackTrack documentation set

Document date: 09 July 2026

Contents

1.  In simple terms

2.  At a glance

3.  How it works — step by step

4.  Where the input comes from

5.  What it produces (output)

6.  What you see on screen

7.  The logic behind it

8.  Detailed technical explanation

9.  Real data vs. synthetic

10.  Use cases

1.  In simple terms

SUMMARY Opens a live connection to the switch and tells you how many ports are free versus in use — and whether an open port is copper (Ethernet) or a fiber SFP slot — so you know instantly whether there is room to plug something in.

2.  At a glance

| Category | Live switch data — reads the real device, not the photo. |
| --- | --- |
| Who uses it | Any technician planning a patch or checking capacity. |
| Where input comes from | A live session to the actual switch's interface table. |
| What it outputs | An availability summary and a filterable per-port inventory. |
| Data source | REAL / LIVE — read from the switch; cached for instant re-open. |

3.  How it works — step by step

Open Ports view for the rack's switch

↓

Probe the switch remote session, interface status requested

↓

Parse interfaces each port → available / in-use / error

↓

Classify medium ETH vs SFP from the switch's data

↓

Show summary + inventory free-of-total, bar, filter pills

From opening the view to a live port inventory.

Walkthrough

Open the Ports view for a rack; an animated loader shows an elapsed-seconds counter while probing.

The server opens a session to the switch and requests its interface status.

Each interface is parsed into a verdict: Available, In use, or Reserved.

Ports are split into Ethernet and SFP sections and a summary card is drawn.

Use the filter pills (All / Available / In Use / Errors) to narrow the list.

If the probe fails, read the reason and press Retry.

4.  Where the input comes from

The switch interface table — the live result of an interface-status command over a remote session.

The switch's medium column — used to tell copper from fiber where the switch reports it.

The scan's port count — a hint for ETH-vs-SFP classification when the switch is quiet.

Configured switch address — the management host to connect to (a fallback is used if none is set).

5.  What it produces (output)

Availability badge — “N of M” free, with an empty style when nothing is free.

ETH / SFP chips — counts of free copper and free fiber ports.

Utilization bar — used ÷ total as a percentage.

Port inventory — each port's number, interface name, description, medium, status and verdict.

Cached snapshot — the last live read, shown instantly on return.

6.  What you see on screen

Hero summary card — availability badge, ETH/SFP breakdown, “· N in use” note.

Ethernet and SFP sections — grouped, each with a port count.

Per-port rows — zero-padded number, full interface name, description, Copper/Fiber medium.

Status + verdict — the switch's raw status text plus a plain Available / In use / Reserved.

Filter pills — All / Available / In Use / Errors, each with a live count.

Retry on failure — with a readable reason (wrong network, closed session, bad credentials).

7.  The logic behind it

Available — link down and no description.

In use — link up / connected.

Reserved — administratively down, shut, or err-disabled, or a down port that still has a description (a sign it is intentionally held).

Honest capacity — a reserved port never counts as open capacity.

Copper vs fiber — decided from the switch's medium column, else inferred from interface naming or the scan's SFP-cage count.

8.  Detailed technical explanation

Connection & parsing

Remote shell — the server opens a secure session to the switch's management address and runs a show-interface-status style command.

Tolerant parser — the interface table is parsed across vendor output styles (for example Cisco-like and TP-Link-like) rather than assuming one exact format.

Caching & errors

Local cache — results are cached on the device; a new scan or Retry triggers a fresh probe.

Friendly errors — raw protocol errors are mapped to short human explanations.

9.  Real data vs. synthetic

| Port up/down / in-use state | REAL / LIVE — from the switch's interface table. |
| --- | --- |
| ETH vs SFP classification | REAL — from the switch's medium / naming, or the scan's port count. |
| Cached last-known state | REAL — the previous live read, re-probed on demand. |
| Fallback host | used only when no switch address is configured. |

10.  Use cases

Planning a patch

Confirm a free copper port and its interface name before running a cable.

Capacity check

The utilization bar shows at a glance whether a switch is nearly full.

Spotting reserved ports

The Errors filter reveals ports that are down on purpose, so they are not mistaken for spare capacity.

— Available Ports —