LIVE NETWORK DATA

Network View (Live Discovery)

Feature Reference

The one view backed by genuine live network telemetry.

Part of the RackTrack documentation set

Document date: 09 July 2026

Contents

1.  In simple terms

2.  At a glance

3.  How it works — step by step

4.  Where the input comes from

5.  What it produces (output)

6.  What you see on screen

7.  The logic behind it

8.  Detailed technical explanation

9.  Real data vs. synthetic

10.  Use cases

1.  In simple terms

SUMMARY Joins your scanned rack to live network-discovery data to show real port up/down state, VLANs, LLDP/CDP neighbors, and how many MAC addresses each port has learned — and lets you look up where any MAC ended up across the network.

2.  At a glance

| Category | Live network data — authoritative connectivity. |
| --- | --- |
| Who uses it | Engineers verifying live connectivity and hunting endpoints. |
| Where input comes from | Live network discovery of the actual devices, matched to the scan. |
| What it outputs | Per-device cards with live port tables grouped by VLAN, plus MAC lookup. |
| Data source | REAL / LIVE — from network discovery, not the scan or synthesized wiring. |

3.  How it works — step by step

Match devices scan devices ↔ discovered devices

↓

Load a device's ports on expand

↓

Group by VLAN with an untagged bucket

↓

Show state + neighbors + MACs up/down, LLDP/CDP, learned MACs

↓

MAC lookup trace an address across discovery

Joining the scan to live discovery.

Walkthrough

Open Network View; a health pill shows whether discovery is online.

Read the match summary (for example “10/10 live”).

Expand a device card to load its port table.

Filter ports by Live / Up / Down / All.

Click a learned-MAC count to look up where that address lives.

4.  Where the input comes from

Live discovery data — port state, VLANs, neighbors and MAC bindings for the real devices.

The scan's device list — matched to discovered devices to scope the view to the rack.

5.  What it produces (output)

Health & match summary — discovery online/offline and how many devices matched.

Device cards — identity, IP, a ports-up bar and a link count.

Port tables — grouped by VLAN with up/down and admin state.

MAC lookup result — which port an address is learned behind.

6.  What you see on screen

Health pill — “Network View online/offline” (or checking).

Match summary — “X/Y live” for how many scan devices matched discovery.

Device cards — name, class, model/vendor, IP; dimmed if not discovered.

Port table — name, description, up/down arrow with admin-state tooltip.

Learned-MAC chips — clickable counts of active MACs behind a port.

Friendly offline banner — a “being prepared” message instead of raw errors.

7.  The logic behind it

Authoritative state — port state here comes from the network itself, not the photo — the one rack view whose connectivity is verified.

Passive gear hidden — patch panels are omitted because they do not appear as neighbors.

Complements topology — topology shows physical layout with inferred cabling; this shows logical, discovered connectivity.

8.  Detailed technical explanation

Join & read

Match — each scanned device is matched to a discovered device.

Read — its ports, VLAN memberships, neighbor relationships and MAC-to-port bindings are read from the discovery datastore.

Push-in

Seeding — rack data can be pushed into discovery so a freshly scanned rack appears, with each port's up/down mirroring the scan's connected/empty detection until real polling refreshes it.

Bidirectional neighbors — links are represented in both directions so they show consistently from either end.

9.  Real data vs. synthetic

| Port up/down state | REAL / LIVE — from discovery. |
| --- | --- |
| VLANs, LLDP/CDP neighbors | REAL / LIVE — from discovery. |
| Learned-MAC counts & lookup | REAL / LIVE — from discovery's MAC bindings. |
| Device match to the rack | REAL — scan device joined to its discovered counterpart. |

IMPORTANT In this deployment the discovery database can also be populated with a seeded demo fabric for demonstration; when real discovery is connected the same view shows genuine live telemetry. The screen always reflects whatever is in the discovery source.

10.  Use cases

Confirming a link is really up

Discovery shows the true operational state, independent of the photo.

Finding an endpoint

A MAC lookup reveals which port a device is learned behind, across the network.

— Network View (Live Discovery) —