LIVE NETWORK DATA

Port History & Drift

Feature Reference

A live change-log for a monitored switch — what changed, and when.

Part of the RackTrack documentation set

Document date: 09 July 2026

Contents

1.  In simple terms

2.  At a glance

3.  How it works — step by step

4.  Where the input comes from

5.  What it produces (output)

6.  What you see on screen

7.  The logic behind it

8.  Detailed technical explanation

9.  Real data vs. synthetic

10.  Use cases

1.  In simple terms

SUMMARY For a continuously monitored switch, this records every port change over time — link up/down, speed, duplex, admin state, medium, description, and neighbor — so you can spot re-cabling and flapping links, with a timeline and a plain-English change log.

2.  At a glance

| Category | Live network data — continuous telemetry. |
| --- | --- |
| Who uses it | Engineers investigating drift, flaps and re-cabling. |
| Where input comes from | A background poller that checks the switch about once a minute. |
| What it outputs | Current state, a per-port timeline, and a change log of every event. |
| Data source | REAL / LIVE — continuous telemetry stored server-side. |

3.  How it works — step by step

Poll the switch about once per minute

↓

Store a snapshot per-port fields captured

↓

Diff vs previous record only real changes

↓

Show grid + timeline state now, history over a window

↓

Plain-English change log “Link came up”, “Speed 1→10 Gbps”

Continuous polling turned into a readable history.

Walkthrough

Open Drift for a monitored switch; a hero shows a Streaming/Paused status.

Read the interface grid, colored by live state.

Tap a port to open its detail sheet.

Use the Specs tab for current values.

Use the Timeline tab and pick a window (1 hour to 1 week).

Use the History tab for a “value at” table and the plain-English change log.

4.  Where the input comes from

Per-minute snapshots — operational and admin state, speed, duplex, flow control, medium, MAC, description and advertised neighbor for each interface.

The chosen time window — 1 hour, 3 hours, 12 hours, a day, or a week.

5.  What it produces (output)

Interface grid — each port colored by live operational state.

Per-port timeline — colored segments for each period a value held.

Change log — human-readable events with from→to values and timestamps.

“Value at” table — the value that held at each time offset.

6.  What you see on screen

Switch hero — streaming status, model/serial/firmware chips, and a “Poll now” button.

Interface grid — ports colored by state; admin-disabled dimmed; selected highlighted.

Timeline — one bar per field (admin, flow control, operational, speed, LLDP neighbor).

Change log — “Link came up”, “Speed changed 1→10 Gbps”, “LLDP neighbor changed A→B”, with exact timestamps.

Empty/sparse note — explains the poller runs once a minute and history accrues over time.

7.  The logic behind it

Only real changes — an event is recorded only when a value actually changes, so the log is the moments a link flapped, renegotiated speed, or changed neighbor — not a wall of snapshots.

Neighbor color — each neighbor name is hashed to a stable color, so a re-cable shows up instantly as a color change on the timeline.

Built by observation — a first-seen switch looks sparse until several poll cycles pass; the view says so rather than implying nothing happens.

8.  Detailed technical explanation

Polling

Cadence — a server-side poller opens a session to the monitored switch roughly once a minute and stores a snapshot of every interface.

Change detection — each snapshot is diffed against the previous one; a change event is written only on a real difference.

Queries

Windows — time-window queries return the value that held at various offsets, so the timeline and “value at” table draw without re-reading the device.

9.  Real data vs. synthetic

| Port state over time | REAL / LIVE — continuous once-a-minute telemetry. |
| --- | --- |
| Change events | REAL — recorded only when a value actually changed. |
| Neighbor changes | REAL — LLDP neighbor tracked; color changes flag re-cabling. |
| Sparse early history | expected — history accrues only as the poller runs. |

10.  Use cases

Chasing a flapping link

The change log pinpoints exactly when a port went down and came back.

Detecting re-cabling

A neighbor color change on the timeline reveals someone moved a cable.

— Port History & Drift —